Table C-1. Proposed Data Format Reporting Format for Reporting of Groundwater Elevation

| Attribute | Definition | Programs that Require Reporting | Required/Optional[a] | Recommended Reporting Accuracy[b] | Unit/Format | Data Type | List of Options |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Local Well Name | Name or number identifier given by the monitoring agency used to identify the well on a local basis. | CASGEM GeoTracker | Optional | N/A | N/A | Text | N/A |
| State Well Number | State well number provided by the Department of Water Resources. | None | At least one well identifier must be entered for each submittal | N/A | N/A | Text | N/A |
| Site Code | Unique identifier assigned to each well for a relevant program that the program uses to track individual wells within the program’s system. | SGMA CASGEM GeoTracker | At least one well identifier must be entered for each submittal | N/A | N/A | Text | N/A |
| Relevant Program | Name of the program that the master code has been specified from to aid in identifying what program the well is associated with. | None | Required, if Site Code entered | N/A | N/A | Text (selected from list of options) | SGMA/CASGEM GeoTracker SAFER |
| Permit Number | Permitting agency permit number associated with well. | None | Optional | N/A | N/A | Text | N/A |
| Local Permit Agency | Agency who approved permit associated with well. | None | Optional | N/A | N/A | Text | N/A |
| Measurement Date | Date groundwater level measurement was taken. | SGMA CASGEM GeoTracker | Required | N/A | MM/DD/YYYY | Date | N/A |
| Measurement Time | Time groundwater level measurement was taken. | SGMA CASGEM | Required | N/A | hh:mm (PST 24-hour) | Time | N/A |
| Reading at Reference Point (RRP) | Reading of the measurement device taken at the RP of the well. | SGMA[c] CASGEM[c] GeoTracker[c] | Required, unless a No Measurement Code is selected | Tenth of a foot | Feet below RP | Numeric | N/A |
| Reading at Water Surface (RWS) | Measurement of the watermark on a steel tape, equivalent, or electronic sounder after making the groundwater sounding; typically, 0.00 for an electronic sounder. | SGMA[c] CASGEM[c] | Optional, default is 0.00 | Tenth of a foot | Feet | Numeric | N/A |
| Groundwater Elevation (GWE) | Calculated GWE. Calculated as: GWE = RPE - (RRP - RWS) RPE = Elevation of the RP from which depth to water measurements are collected, which is reported as part of the Well Characteristics attributes | None | Optional | Tenth of a foot | Feet; default in NAVD88 | Numeric | N/A |
| Ground Surface (GS) to Water Surface (WS) | Depth to water below ground surface. Calculated as: GS to WS = GSE - WSE GSE = Elevation of the ground surface associated with the location of the well, which is reported as part of the Well Characteristics attributes. | None | Optional | Tenth of a foot | Feet below ground surface | Numeric | N/A |
| Water Level Datum | Vertical datum associated with calculated water surface elevation. | None | Required | N/A | NAVD88 (recommended) | Text (selected from list of options) | WGS84 NAVD88 NGVD29 |
| No Measurement Code | Code providing explanation for no groundwater level measurement being taken. | SGMA[c] CASGEM[c] | Required, unless an RRP is entered | N/A | N/A | Text (selected from list of options) | 0 – Measurement Discontinued 1 – Pumping 2 – Pump house locked 3 – Tape hung up 4 – Can't get tape in casing 5 – Unable to locate well 6 – Well has been destroyed 7 – Special/Other 8 – Casing leaking or wet 9 – Temporarily inaccessible D – Dry well F – Flowing artesian well |
| Measurement Quality Code | Code providing a description of the quality of the groundwater level measurement. | None | Optional | N/A | N/A | Text (selected from list of options) | Caved or deepened Pumping Nearby pump operating Casing leaking or wet Pumped recently Air or pressure gauge measurement Other Recharge or surface water effects near well Oil or foreign substance in casing Acoustical sounder Recently flowing Flowing Nearby flowing Nearby recently flowing Dry Injector site monitor Injector site monitor Plugged Measurement discontinued Obstruction Nearby recently pumped Foreign substance Well destroyed Surface water effects Blank – static |
| Measurement Method | Method used to collect groundwater level measurement. | SGMA[c] CASGEM[c] | Required, if RRP is entered | N/A | N/A | Text (selected from list of options) | Airline Acoustic or sonic sounder Analog or graphic recorder Calibrated airline Estimated Electric sounder measurement Transducer Pressure-gage Calibrated pressure-gage Interpreted from geophysical logs Manometer Non-recording gage Observed Reported, method unknown Steel-tape measurement Electric-tape Electronic pressure transducer Calibrated electric tape Other Unknown |
| Measurement Accuracy | Accuracy of groundwater level measurement. | SGMA[c] CASGEM[c] | Optional | N/A | N/A | Text (selected from list of options) | 1 foot 0.1 foot 0.01 foot 0.001 foot Not to nearest foot Unknown Other |
| Collection Agency | Full name of collection or co-op agency that collected groundwater level measurement. | SGMA CASGEM | Optional | N/A | N/A | Text | N/A |
| Reporting Agency | Full name of reporting agency that reports the groundwater level measurement. | None | Optional | N/A | N/A | Text | N/A |
| Water Level Comments | General description or other notes associated with groundwater level measurement. | SGMA[c] | Optional | N/A | N/A | Text | N/A |

[a]Attributes designated as optional may be required depending on program specific reporting compliance. Fields designated as required are deemed to be the most important attributes for reporting. However, if required data are missing, the user may still upload the attributes that are available. Required fields that are not submitted will cause the data to be flagged for quality control purposes. Additional evaluation of the SAFER program’s level of requirements is still needed to highlight program reporting requirements.

[b]The Recommended Reporting Accuracy does not preclude data outside of the recommended accuracy from being submitted. Data with Unknown or Other Accuracy may be flagged as potentially inaccurate data.

[c] Designated as a conditional reporting requirement for the specific program.

CASGEM = California Statewide Groundwater Elevation Monitoring

GeoTracker = GeoTracker Data Management System

N/A = not applicable

NAVD88 = North American Vertical Datum of 1988

NGVD29 = North Geodetic Vertical Datum of 1929
PST = Pacific Standard Time
RP = reference point

RPE = reference point elevation
RRP = reading at reference point
RWS = reading at water surface

WGS84 = World Geodetic System 
WSE = water surface elevation

SAFER = Safe and Affordable Funding for Equity and Resilience Program

SGMA = Sustainable Groundwater Management Act