Table 3A1-1. Comparison of the Proposed Data Reporting Format to Existing Groundwater Elevation Reporting Programs

| Attribute | Proposed Data Reporting Format | Sustainable Groundwater Management Act Program | California Statewide Groundwater Elevation Monitoring (CASGEM) Program | State Water Resources Control Board GeoTracker Data Management System | US Geologic Survey National Water Information System | SAFER |
| --- | --- | --- | --- | --- | --- | --- |
| Local Well Name | Attribute: Local Well Name Definition: Name or number identifier given by the monitoring agency used to identify the well on a local basis. | Attribute: Local Well Name Definition: Local well name used to identify the well. | Attribute: Local or State Well Number Definition: Name or number identifier given to the well by the monitoring entity. | Attribute: Field Point Name Definition: Field name or common name of the location where field measurement was collected. | Attribute: Site/Station Name Definition: Site names generally include river and lake names. For wells it can be the local number. | Facility Name Facility ID |
| State Well Number | Attribute: State Well Number Definition: State well number provided by the Department of Water Resources. | Attribute: State Well Number Definition: Official State Well Number provided by the Department of Water Resources or local agency that has been granted authority to assign the number | Attribute: State Well Number Definition: Official State Well Number provided by the Department of Water Resources or local agency that has been granted authority to assign the number. | Not reported. | Not reported. | Not reported. |
| Site Code | Attribute: Site Code Definition: Unique identifier assigned to each well for relevant program that the program uses to track individual wells within its system. | Attribute: Site Code Definition: Unique site code created for the well by the Monitoring Network Module or CASGEM. | Attribute: Site Code / CASGEM ID Definition: Unique identifier assigned and used by the CASGEM system. | Attribute: Global ID Definition: Unique identifier for a regulated facility or site. | Attribute: Site Number/ID – 1 Definition: 5-digit number ID for wells (8-digit number ID is for surface water). | Well Completion Report Number Department of Water Resources Site Code Identification Number Water Rights ID |
| Permit Number | Attribute: Permit Number Definition: County permit number associated with well. | Not reported. | Not reported. | Not reported. | Not reported. | Well Permit Number |
| Local Permit Agency | Attribute: Local Permit Agency Definition: Agency who approved permit associated with well. | Not reported. | Not reported. | Not reported. | Not reported. | Not reported. |
| Measurement Date/Time | Attribute: Measurement Date Definition: Date groundwater level measurement was taken. Date Format: MM/DD/YYYY Attribute: Measurement Time Definition: Time groundwater level measurement was taken. Time Format: hh:mm:ss 24-hour time Pacific Standard Time (PST) (default time is 12:00:00). | Attribute: Measurement Date Definition: (MM/DD/YYYY) – Groundwater level measurement date. Attribute: Measurement Time (PST 24-hour) Definition: Time when the groundwater level measurement was taken. | Date (MM/DD/YYYY) 24-hour Time, PST (hh:mm) | Date on which the elevation was measured. Date format MM/DD/YYYY | Date – Date of measurement (MM/DD/YYYY). Time – 00:00 24-hour format, provide standard time zone code as well as if it is in daylight savings time. | Attribute: Static Water Level Date Measured Definition: The date that the static water level measurement for a groundwater or ground water under direct influence (GWUDI) source was measured.​ Not Available option provided. MM/DD/YYYY. Attribute: Pumping Water Level Date Measured Definition: The date that the pumping water level measurement for a groundwater or GWUDI source was measured. MM/DD/YYYY. |
| Reading at Reference Point (RRP) | Attribute: RRP Definition: Reading of the measurement device taken at the reference point of the well. Units: Feet Reporting Accuracy: Tenth of a foot. | Attribute: RRP (feet) Definition: Reading of measurement device taken at the reference point of the well. | Attribute: RRP Definition: Reading of the measurement device taken at the reference point of the well. Enter a numerical value in feet. | Attribute: DTW Definition: Measured depth from top of well casing to groundwater surface reported to hundredth of a foot. | Attribute: Water Level Definition: Enter the water level, in feet from measuring point at the site. Water levels below measuring point are unsigned, water levels above monitoring point are preceded by a minus sign (-), if water level type is M (measuring point). Precision can be carried to two decimal places. | Not reported. |
| Reading at Water Surface (RWS) | Attribute: RWS Definition: Measurement of the watermark on a steel tape, equivalent, or electronic sounder after making the groundwater sounding, which is typically 0.00 for an electronic sounder. Units: Feet Reporting Accuracy: Tenth of a foot | Attribute: RWS (feet) Definition: The reading of the measurement device taken at the water surface (WS) of the well. Enter a numerical value in feet to the tenth decimal. | Attribute: RWS Definition: The reading of the measurement device taken at the WS of the well. Enter a numerical value in feet. Value must be between -300 and 14,505. | Not reported. | Not reported. | Not reported. |
| Reading at Vertical Datum | Attribute: Reading at Vertical Datum Definition: Groundwater level measurement reading as depth to water from the measurement point below the vertical datum. Negative value is considered above the vertical datum. Units: Feet Reporting Accuracy: Two decimal points. | Not reported. | Not reported. | Not reported. | Attribute: Water Level Definition: Enter the water level, in feet from measuring point at the site. Water levels below vertical datum are unsigned, water levels above vertical datum are preceded by a minus sign (-), if water level type is S (vertical datum). Precision can be carried to two decimal places. | Not reported. |
| Groundwater Elevation (GSE) (Calculated) | Attribute: GWE Definition: Calculated GWE. Calculated as: GWE = RPE - (RRP or RWS) RPE = reference point elevation which is reported as part of the well characteristic attributes. Units: Feet above North American Vertical Datum of 1988 (NAVD 88). | Attribute: Groundwater Elevation Definition: Groundwater elevation in feet above mean sea level, using NAVD 88. | Attribute: WSE Definition: A calculated field with no input required. | Not reported. | Not reported. | Not reported. |
| Ground Surface (GS) to WS (Reported or Calculated) | Attribute: GS to WS Definition: Depth to water below ground surface (bgs). Calculated as: GS to WS = GSE – WSE Units: Feet bgs Reporting Accuracy: Tenth of a foot | Attribute: GS Elevation to Groundwater Elevation Definition: Depth to groundwater elevation in feet bgs. | Attribute: GS to WS Definition: Distance measured from GS to WS, or DTW. A calculated field, no is input required. | Not reported. | Attribute: Water Level Definition: Enter the water level, in feet from land surface datum (LSD) at the site. Water levels below LSD are unsigned, water levels above LSD are preceded by a minus sign (-), if water level type is L (land surface). Precision can be carried to two decimal places. | Attribute: Static Water Level Definition: Feet bgs. Used for groundwater and GWUDI sources only, this identifies the number of feet from the ground surface to the water table. This measurement is not applicable for horizontal wells.​ Attribute: Pumping Water Level Definition: Feet bgs. Identifies the water level of a groundwater or GWUDI source while that source is actively producing water in a measurement of feet from the ground surface to the water table during pumping. |
| Reading Point (RP) to Water Level (Calculated) | Attribute: RP to Water Level Definition: Depth to water below reading point (bgs). Calculated as: RP to water level = RP – water level Units: Feet bgs Reporting Accuracy: Tenth of a foot. | Attribute: Reference Point Elevation to Groundwater Elevation Definition: Depth to groundwater elevation in feet below reference point. | Attribute: RP to WS Definition: Calculated value, no input required. Reading at RP - Reading at WS. | Refer to RRP. | Not reported. | Not reported. |
| Water Level Datum | Attribute: Water Level Datum Definition: Vertical datum of WSE. Format: NAVD 88 (recommended) | Attribute: Water Level Datum Definition: Water level datum as NAVD 88. | Attribute: Water Level Datum Definition: Water level datum as NAVD 88. | Not reported. | Attribute: Water level Datum Definition: [depth to] water level datum, mandatory if water level type is S (vertical datum) or other. | Not reported. |
| No Measurement Code | Attribute: No Measurement Code Definition: Code providing explanation for no groundwater level measurement being taken. List of Options: 0 – Measurement discontinued, 1 – Pumping, 2 – Pump house locked, 3 – Tape hung up, 4 – Can’t get tape in casing, 5 – Unable to locate well, 6 – Well has been destroyed, 7 – Special/Other, 8 – Casing leaking or wet, 9 – Temporarily inaccessible, D - Dry well, F - Flowing artesian well | Attribute: No Measurement Type/Code Definition: Type/code used to explain why a measurement was not taken. 0 – Measurement discontinued, 1 – Pumping, 2 – Pump house locked, 3 – Tape hung up, 4 – Can’t get tape in casing, 5 – Unable to locate well, 6 – Well has been destroyed, 7 – Special/Other, 8 – Casing leaking or wet, 9 – Temporarily inaccessible, D – Dry well, F – Flowing artesian well | Attribute: No Measurement Code Definition: Code to explain why a measurement was not taken. 0 – Measurement discontinued, 1 – Pumping, 2 – Pump house locked, 3 – Tape hung up, 4 – Can’t get tape in casing, 5 – Unable to locate well, 6 – Well has been destroyed, 7 – Special/Other, 8 – Casing leaking or wet, 9 – Temporarily inaccessible, D – Dry well, F – Flowing artesian well | Not reported. | Not reported. | Not reported. |
| Measurement Quality Code | Attribute: Measurement Quality Code Definition: Code providing a description of the quality of the groundwater level measurement. List of Options: Caved or deepened Pumping Nearby pump operating Casing leaking or wet Pumped recently Air or pressure gauge measurement Other Recharge or surface water effects near well Oil or foreign substance in casing Acoustical sounder Recently flowing Flowing Nearby flowing Nearby recently flowing Dry Injector site monitor Injector site monitor nearby Plugged Measurement discontinued Obstruction Nearby recently pumped Foreign substance Well destroyed Surface water effects Blank – Static | Attribute: Questionable Measurement Type Code Definition: Type/code to explain why a measurement is questionable. 0 – Caved or deepened, 1 – Pumping, 2 – Nearby pump operating, 3 – Casing leaking or wet, 4 – Pumped recently, 5 – Air or pressure gauge measurement, 6 – Other, 7 – Recharge or surface water effects near well, 8 – Oil or foreign substance in casing, 9 – Acoustical sounder, E – Recently flowing, F – Flowing, G – Nearby flowing, H – Nearby recently flowing | Attribute: Questionable Measurement Code Definition: Code to explain why a measurement is questionable 0 – Caved or deepened, 1 – Pumping, 2 – Nearby pump operating, 3 – Casing leaking or wet, 4 – Pumped recently, 5 – Air or pressure gauge measurement, 6 – Other, 7 – Recharge or surface water effects near well, 8 – Oil or foreign substance in casing, 9 – Acoustical sounder, E – Recently flowing, F – Flowing, G – Nearby flowing, H – Nearby recently flowing | Not reported. | Attribute: Site Status for Water Level Definition: Enter the code that best describes the status of the site or water level at the time the water level was measured. If no status is indicated, the reported water level measurement is assumed to represent a static water level. D – Dry E – Recently flowing F – Flowing G – Nearby flowing H – Nearby recently flowing I – Injector site monitor J – Injector site monitor nearby M – Plugged N – Measurement discontinued O – Obstruction P –Pumping R – Recently pumped S – Nearby pumped T – Nearby recently pumped V – Foreign substance W – Well destroyed X – Surface water effects Z – Other Blank – Static | Not reported. |
| Measurement Method | Attribute: Measurement Method Definition: Method used to collect groundwater level measurement. List of Options: Airline Acoustic or sonic sounder Analog or graphic recorder Calibrated airline Estimated Electric sounder measurement Transducer Pressure gauge Calibrated pressure gauge Interpreted from geophysical logs Manometer Non-recording gauge Observed Reported, method unknown Steel-tape measurement Electric tape Electronic pressure transducer Calibrated electric tape Other Unknown | Attribute: Measurement Method Type/Code Definition: Measurement method type/code used to collect groundwater level measurement. ES – Electric sounder measurement ST – Steel tape measurement AS – Acoustic or sonic sounder PG – Airline measurement, pressure gauge, or manometer TR – Electronic pressure transducer OTH – Other UNK – Unknown | Attribute: Measurement Method Code Definition: Specify the method by which water levels were measured. ES – Electric sounder measurement ST – Steel tape measurement AS – Acoustic or sonic sounder PG – Airline measurement, pressure gauge, or manometer TR – Electronic pressure transducer OTH – Other UNK – Unknown | Not reported. | Attribute: lev_meth_cd Definition: Code indicating how the water level was measured. When a water level is entered, the code is mandatory A – Airline B – Analog or graphic recorder C – Calibrated airline E – Estimated F – Transducer G – Pressure gauge H – Calibrated pressure gauge L – Interpreted from geophysical logs M – Manometer N – Non-recording gauge O – Observed R – Reported, method unknown S – Steel tape T – Electric tape V – Calibrated electric tape Z – Other (explain in Miscellaneous Remarks) Attribute: Method of Water Level Measurement Definition: Enter code that best describes how the water level was measured. Attribute: Water level Type Code Definition: Type of water level measured: L (land surface), M (measure point), or S (vertical datum) | Not reported. |
| Measurement Accuracy | Attribute: Measurement Accuracy Definition: Accuracy of groundwater level measurement. Includes a list of options to select from. | Attribute: Measurement Accuracy Definition: Accuracy of groundwater level measurement. | Attribute: Measurement Accuracy Definition: Estimated accuracy of the groundwater level measurement. | Hundredth of a foot. | Attribute: lev_acy_cd Definition: Code indicating accuracy of the water level measurement. 0 – Nearest foot 1 – Nearest tenth of a foot 2 – Nearest one-hundredth of a foot 9 – Not to nearest foot U – Unknown Attribute: Water Level Accuracy Definition: Foot, tenth, hundredth, or not to nearest foot | Not reported. |
| Collection Agency | Attribute: Collection Agency Definition: Full name of collection or co-op agency that collected groundwater level measurement. | Attribute: Collecting/Co-op Agency Definition: Full name of the collecting/co-op agency that collected the groundwater level measurement. | Attribute: Collecting Agency Definition: Full name of the agency that collected the groundwater level measurement. | Not reported. | Attribute: Agency Code Definition: Five-character code for the agency that is the source of data for a site or that is responsible for entering into GWSI Attribute: Person Making Measurement Definition: Six-character text field of the person making the water level measurement Attribute: Measuring Agency Definition: Five-character code for the person who measured the water level | Not reported. |
| Reporting Agency | Attribute: Reporting Agency Definition: Full name of reporting agency that reports the groundwater level measurement. | Attribute: Reporting Agency Definition: Full name of reporting agency that reports the groundwater level measurement. | Not reported. | Not reported. | Not reported. | Not reported. |
| Water Level Comments | Attribute: Water Level Comments Definition: General description or other notes associated with groundwater level measurement. | Attribute: Water Level Measurement Comments Definition: Use this field to explain special circumstances related to the water level measurement not indicated in other fields. | Attribute: Comments Definition: Use this field to explain special circumstances related to the water level measurement not indicated in other fields. Also specify whether the information previously submitted or overwrites existing data. | Attribute: GW_MEAS_DESC Definition: DTW general description. | Not reported. | Not reported. |

Multiple reporting fields may be included in an attribute based on the program’s reporting format.

CASGEM = California Statewide Groundwater Elevation Monitoring

DTW = depth to water

GS = ground surface

LSD = land surface datum

NAVD88 = North American Vertical Datum of 1988
PST = Pacific Standard Time
RP = reference point

RPE = reference point elevation
RRP = reading at reference point
RWS = reading at water surface

SAFER= Safe and Affordable Funding for Equity and Resilience Program

WS = water surface
WSE = water surface elevation