Table 3B-1. Comparison of the Proposed Data Reporting Format to Existing Well Characteristics Reporting Programs

| Attribute | Proposed Data Reporting Format | Sustainable Groundwater Management Act Program | California Statewide Groundwater Elevation Monitoring (CASGEM) Program | State Water Resources Control Board GeoTracker Data Management System | US Geologic Survey (USGS) National Water Information System | SAFER | Online System of Well Completion Reports |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Local Well Name | Attribute: Local Well Name Definition: Name or number identifier given by the monitoring agency used to identify the well on a local basis. | Attribute: Local Well Name Definition: Local well name used to identify the well. | Attribute: Local or State Well Number Definition: Name or number identifier given to the well by the monitoring entity. | Attribute: Field Point Name Definition: Field name or common name of the location where field measurement was collected. | Attribute: Site/Station Name Definition: Site names generally include river and lake names. For wells it can be the local number. | Facility Name Facility ID | Attribute: Owner’s Assigned Well Number Definition: Owner-assigned well number. |
| State Well Number | Attribute: State Well Number Definition: State Well Number provided by the Department of Water Resources. | Attribute: State Well Number Definition: State Well Number provided by the Department of Water Resources. | Attribute: Local or State Well Number Definition: Name or number identifier given to the well by the monitoring entity. | Not reported. | Not reported. | Department of Water Resources Site Code Identification Number Water Rights ID | State Well Number/‌Site Number |
| Site Code | Attribute: Site Code Definition: Unique identifier assigned to each well for a relevant program that the program uses to track individual wells within its system. | Attribute: Site Code Definition: Unique site code created for the well by the monitoring network module or CASGEM. | Attribute: Master Site Code Definition: Unique identifier assigned and used by the CASGEM system. | Attribute: Global ID Definition: Unique identifier for a regulated facility or site | Attribute: Site Number/ID Definition: 15-digit number ID for wells (8-digit number ID is for surface water). | Not reported. | Site Code |
| Permit Number | Attribute: Permit Number Definition: County permit number associated with well. | Not reported. | Not reported. | Not reported. | Not reported. | Well Permit Number | Attribute: Permit Number Definition: Well permit number |
| Local Permit Agency | Attribute: Local Permit Agency Definition: Agency who approved permit associated with well. | Not reported. | Not reported. | Not reported. | Not reported. | Not reported. | Attribute: Local Permit Agency Definition: Local regulatory authority over well construction, alteration, and destruction. |
| Latitude | Attribute: Latitude Definition: Latitude of the well location (Y Coordinate). Units: Decimal degrees Reporting Accuracy: 5 decimal places (36 feet). | Attribute: Latitude Definition: Latitude of the well location, reported in decimal degrees to five decimal places, to a minimum accuracy of 30 feet, relative to NAD83 or another national standard that is convertible to NAD83. | Attribute: Latitude [N] Definition: positive value between 32 and 42.5. | Attribute: Latitude Definition: The latitude (Y coordinate) of the survey point, measured in decimal degrees, and reported to 7 decimal points. | Attribute: Latitude Definition: The latitude in NAD83 for the Well Site | Facility Latitude | Attribute: Decimal Latitude Definition: Decimal latitude, converted from text to number. |
| Longitude | Attribute: Longitude Definition: Longitude of the well location (X Coordinate) Units: decimal degrees Reporting Accuracy: 5 decimal places (36 feet) | Attribute: Longitude Definition: Longitude of the well location, reported in decimal degrees to five decimal places, to a minimum accuracy of 30 feet, relative to NAD83 or another national standard that is convertible to NAD83. | Attribute: Longitude [W] Definition: negative value between −124.5 and −114 | Attribute: Longitude Definition: The longitude (X coordinate) of the survey point, measured in decimal degrees, and reported to 7 decimal points. | Attribute: Longitude Definition: The longitude in NAD83 for the well site. | Facility Longitude | Attribute: Decimal Longitude Definition: Decimal longitude, converted from text to number. |
| Coordinates Survey Date | Attribute: Coordinate Survey Date Definition: Date that the XY coordinates of the well were surveyed. Date Format: MM/DD/YYYY | Not reported. | Not reported. | Attribute: XY_SURVEY_DATE Definition: The date on which the latitude & longitude coordinates were measured | Not reported. | Not reported. | Attribute: Date Work Ended Definition: Date on which work on well ended, converted from text to date. |
| Coordinate Survey Time | Attribute: Coordinate Survey Time Definition: Time that the XY coordinates of the well were surveyed. | Not reported. | Not reported. | Not reported. | Not reported. | Not reported. | Not reported. |
| Coordinates Datum | Attribute: Coordinates Datum Definition: Horizontal datum that the XY coordinates were measured. Format: NAD83 (recommended). List of Options: NAD83, NAD27, WGS84 | Not reported. | Not reported. | Attribute: XY_DATUM Definition: The code representing the datum from which the latitude/longitude coordinates were determined. | Attribute: Latitude and Longitude Datum Definition: NAD27 or NAD83. | Facility Latitude and Longitude Datum | Attribute: Horizontal Datum Definition: Datum of horizontal coordinates. |
| Coordinates Method | Attribute: Coordinates Method Definition: Method used to determine latitude and longitude of the well. List of Options: Surveyed USGS quad GPS GPS with WAAS Digitized from aerial photo Unknown Other | Attribute: Coordinates Method List of Options: Surveyed USGS quad GPS GPS with WAAS Digital aerial photo Unknown Other | Attribute: Coordinates Method List of Options: Surveyed USGS quad GPS GPS with WAAS Digital aerial photo Unknown Other | Attribute: XY_METHOD Definition: The code representing the survey method by which the latitude/longitude measurements were collected. | Attribute: Latitude and Longitude Method Definition: Code best describing the method used to determine latitude and longitude. | Not reported. | Attribute: Method of Determination LL Definition: Method used to determine horizontal coordinates. |
| Coordinates Accuracy | Attribute: Coordinates Accuracy Definition: Accuracy of well location latitude and longitude measurements. | Coordinates Accuracy List of Options: 1 foot 10 feet 30 feet 75 feet 150 feet Unknown | Coordinates Accuracy List of Options: 1 foot 10 feet 30 feet 75 feet 150 feet Unknown | Attribute: XY_ACC_VAL Definition: The accuracy range (+/-) of the latitude and longitude reported in centimeters at a 95% confidence interval. | Attribute: Latitude and Longitude Accuracy Definition: code that best describes the accuracy of the latitude and longitude. | Not reported. | Attribute: LL Accuracy Definition: Accuracy of horizontal coordinates. |
| Coordinates Surveyor | Attribute: Coordinates Surveyor Definition: The full names of the organization and surveyor who collected the latitude/longitude coordinates. | Attribute: Survey Organization Definition: The organization that completed the Survey. | Attribute: Survey Organization Definition: The organization that completed the Survey. | Attribute: XY_SURVEY_ORG Definition: The name of the organization who collected the latitude/longitude coordinates. | Not reported. | Not reported. | Not reported. |
| Survey Type | Attribute: Survey Type Definition: The general description for the Survey. The initial Survey associated with a new well is always labeled Initial Elevation. List of options: - Initial Elevation - Increased Accuracy - Reference Point Elevation (RPE) Change Other | Attribute: Survey Type Definition: The general description for the Survey. The initial Survey associated with a new well is always labeled Initial Elevation. List of options: - Initial Elevation - Increased Accuracy - Reference Point Elevation (RPE) Change - Other | Attribute: Survey Type Definition: The general description for the Survey. The initial Survey associated with a new well is always labeled Initial Elevation. List of options: - Initial Elevation - Increased Accuracy - Reference Point Elevation (RPE) Change Other | Not reported. | Not reported. | Not reported. | Not reported. |
| Coordinates Equipment | Attribute: Coordinates Equipment Definition: The name or type of GPS equipment and model number used to collect the latitude/longitude coordinates. | Not reported. | Not reported. | Attribute: GPS_EQUIP_TYPE Definition: The name of the GPS unit used to determine the latitude/longitude coordinates. Left blank if survey method did not involve GPS | Not reported. | Not reported. | Not reported. |
| Coordinates Survey Field Notes | Attribute: Coordinates Survey Field Notes Definition: Any additional notes from the field during coordinates survey. | Not reported. | Not reported. | Attribute: XY_SURVEY_DESC Definition: General description information pertaining to the survey of latitude/‌longitude. May describe offset azimuth, distance, and slope | Not reported. | Not reported. | Not reported. |
| Reference Point Elevation (RPE) | Attribute: RPE Definition: Elevation of the reference point from which the groundwater level measurements are collected. Units: Feet NAVD88 (recommended) Reporting Accuracy: Tenth of a foot or greater accuracy. | Attribute: RPE (feet) Definition: elevation of the well’s reference point. Elevations shall be measured and reported in feet to an accuracy of at least 0.5 foot. | Attribute: RPE Definition: Elevation of the reference point for water level measurements. Enter a numerical value in feet (NAVD88). | Attribute: ELEVATION Definition: Elevation of the survey point measured to top of well casing to a hundredth of a foot between well locations within the site. | Not reported. | Not reported. | Not reported. |
| Reference Point Description | Attribute: Reference Point Description Definition: Description of the reference point location on the well and any other relevant notes regarding the well’s reference point from which groundwater level measurements are collected. | Attribute: Reference Point Description Definition: Description of the location of the well’s reference point and any other notes used to identify the well’s reference point. | Not reported. | Not reported. | Not reported. | Not reported. | Not reported. |
| Ground Surface Elevation (GSE) | Attribute: GSE Definition: Elevation of the ground surface associated with the location of the well. Units: Feet NAVD88 (recommended). Reporting Accuracy: Tenth of a foot or greater accuracy. | Attribute: GSE (feet) Definition: Elevation of the ground surface at the well. Elevations shall be measured and reported in feet to an accuracy of 0.1 foot (NAVD 88). | Attribute: GSE Definition: Elevation of the ground surface at the well. Enter a numerical value in feet (NAVD 88). | Not reported. | Attribute: Altitude Definition: Land surface elevation above NAVD88. | Not reported. | GSE |
| Elevation Survey Date | Attribute: Elevation Survey Date Definition: Date that the RPE and/or ground surface elevation was surveyed. Date Format: MM/DD/YYYY | Attribute: Start Date Definition: The date that the Survey was completed Date Format: MM/DD/YYYY | Attribute: Start Date Definition: The date that the Survey was completed Date Format: MM/DD/YYYY | Attribute: ELEV_SURVEY_DATE Definition: date on which the elevation was measured. | Attribute: Date Definition: Date of measurement (MM/DD/YYYY). Attribute: Time Definition: 00:00 24-hour format, provide standard time zone code as well as if it is in daylight savings time. | Not reported. | Not reported. |
| Elevation Survey Time | Attribute: Elevation Survey Time Definition: Time that the reference point elevation and/or ground surface elevation was surveyed. | Not reported. | Not reported. | Not reported. | Not reported. | Not reported. | Not reported. |
| Elevation Measurement Method | Attribute: Elevation Measurement Method Definition: Method used to survey elevations. List of Options: Altimeter. Differentially corrected GPS. Survey-grade GPSF. Interferometric synthetic aperture radar, airplane. Light detection and ranging, airplane. Level or other surveyed method. Interpolated from topographic map. Interpolated from digital elevation model. Reported method of determination. GNSS1 – Level 1 Quality Survey-grade Global Navigation Satellite System. GNSS2 – Level 2 Quality Survey-grade Global Navigation Satellite System. GNSS3 – Level 3 Quality Survey-grade Global Navigation Satellite System. GNSS4 – Level 4 Quality Survey-grade Global Navigation Satellite System. Surveyed to a benchmark. USGS quad. GPS. GPS with WAAS. Unknown. Other. | Attribute: Elevation Method Definition: Method used to define ground surface and reference point elevation. List of Options: Surveyed to a benchmark USGS quad GPS GPS with WAAS Unknown Other | Not reported. | Attribute: ELEV_METHOD Definition: Code representing the method by which the elevation measurement was collected. List of Options: DIG – Conventional differential leveling, digital or non-digital (Federal Geodetic Control Subcommittee, Third Order) based on minimum of 2 geodetic control points. TRIG – Trigonometric leveling using total stations (Federal Geodetic Control Subcommittee, Third Order) based on minimum of 2 geodetic control. (May not achieve 0.1 foot accuracy; may not be compliant with AB2886. Results will vary with length of line and accuracy of equipment utilized.) CGPS – Conventional elevation survey technique (Federal Geodetic Control Subcommittee Third Order) based on minimum of 2 GPS-derived control points (RTK, static, and rapid/fast-static) established from HPGN or HPGN-densification geodetic control points. | Attribute: Altitude Method Definition: Code that best describes method used to determine altitude. List of Options: Altimeter. Differentially corrected GPS. Survey-grade GPSF. Interferometric synthetic aperture radar, airplane. Light detection and ranging, airplane. Level or other surveyed method. Interpolated from topographic map. Interpolated from digital elevation model. Reported method of determination. GNSS1 – Level 1 Quality Survey-grade Global Navigation Satellite System. GNSS2 – Level 2 Quality Survey-grade Global Navigation Satellite System. GNSS3 – Level 3 Quality Survey-grade Global Navigation Satellite System. GNSS4 – Level 4 Quality Survey-grade Global Navigation Satellite System. | Not reported. | Attribute: Elevation Determination Method Definition: Method used to determine elevation. |
| Elevation Vertical Datum | Attribute: Elevation Vertical Datum Definition: Vertical datum associated with elevation measurements. Format: NAVD88 (recommended). Reporting Accuracy: Tenth of a foot or greater accuracy. | NAVD88 | NAVD88 | Attribute: ELEV_DATUM Definition: Code representing datum from which elevation was determined. NAVD88 preferred. | Attribute: Altitude Datum Definition: NGVD29 or NAVD88 | Not reported. | Attribute: Vertical Datum Definition: Datum of vertical coordinate/‌elevation. |
| Elevation Accuracy | Attribute: Elevation Accuracy Definition: Accuracy of elevation measurements. List of Options: 0.1 feet 0.01 feet 0.001 feet 1 foot 2.5 feet feet 10 feet 50 feet 50 feet Unknown | Attribute: Elevation Accuracy Definition: accuracy of the elevation method used to define ground surface and reference point elevation. List of Options: 0.1 foot 2.5 feet 5 feet 10 feet 20 feet 50 feet > 50 feet Unknown | Attribute: Elevation Accuracy Definition: accuracy of the elevation method used to define ground surface and reference point elevation. List of Options: 0.1 foot 2.5 feet 5 feet 10 feet 20 feet 50 feet > 50 feet Unknown | Attribute: ELEV_ACC_VAL Definition: Accuracy range of the absolute elevation measurement report in centimeters at the 95% confidence interval. | Attribute: Altitude Accuracy Definition: Code that best describes accuracy of altitude in terms of possible error, in feet. | Not reported. | Attribute: Elevation Accuracy Definition: Accuracy of elevation. |
| Elevation Surveyor Name | Attribute: Elevation Surveyor Name Definition: The name of the organization collecting the elevation measurements. | Not reported. | Not reported. | Attribute: ELEV_SURVEY_ORG Definition: The name of the organization collecting the elevation measurement. | Not reported. | Not reported. | Not reported. |
| Elevation Survey Field Notes | Attribute: Elevation Survey Field Notes Definition: Any additional notes from the field during elevation survey | Not reported. | Not reported. | Attribute: ELEV_DESC Definition: General description information pertaining to the survey. | Not reported. | Not reported. | Not reported. |
| Well Riser Height (RHT) | Attribute: Well Riser Height (RHT) Definition: Measured distance from ground surface to top of well casing. Calculated as: RHT = RPE – GSE. Reporting Accuracy: Tenth of a foot or greater accuracy. | Automatically calculated (not reported). | Automatically calculated (not reported). | Attribute: RISER_HT Definition: Measured distance from ground surface to top of well casing reported as a positive or negative value to a hundredth of a foot. | Not reported. | Not reported. | Not reported. |
| Total Well Depth | Attribute: Total Well Depth Definition: Depth to the bottom of the well below ground surface. Units: Feet below GSE Reporting Accuracy: Tenth of a foot or greater accuracy. | Attribute: Well Depth Definition: A well’s total casing depth or open borehole depth below ground surface in feet. | Not reported. | Attribute: TOT_DEPTH Definition: Depth of well measured in the field during the sampling event from top of well casing to “bottom” of well reported to hundredth of a foot. | Attribute: Well Depth Definition: Depth of well in feet below land surface. Hole Depth Definition: Total depth to which the hole is drilled in feet below land surface. | Attribute: Well Depth Definition: Feet from surface. | Attribute: Total Drill Depth Definition: Total drill depth. Attribute: Total Completed Depth Definition: Depth to which well was completed, converted from text to number. |
| Type of Opening | Attribute: Type of Opening Definition: How water enters the well from the aquifer. Most drilling methods use a well screen while cable tooled wells may use an open hole. List of Options: Fractured rock Louvered or shutter type Mesh screen Perforated, porous, slotted Wire-wound screen Screen, type unknown Sand point screen Walled or shored Open hole Other | Not reported. | Not reported. | Not reported. | Attribute: Type of Opening Definition: Enter the code that best describes the type of openings in an open interval. List of Options: Fractured rock Louvered or shutter type Mesh screen Perforated, porous, slotted Wire-wound screen Screen, type unknown Sand point screen Walled or shored Open hole Other | Not reported. | Casing Type |
| Depth to Top of Screen (1)…(10) | Attribute: Depth to Top of Screen (1)…(10) Definition: Depth to the top of a screened interval below ground surface. Can include up to 10 different screened intervals. Units: Feet below GSE Reporting Accuracy: tenth of a foot or greater | Attribute: Top Perforation Definition: Depth below ground surface to the top of a perforation interval measured in feet. | Attribute: Screen Interval Top Definition: Top screen interval. | Not reported. | Attribute: Depth to Top of Interval Definition: Enter the depth, in feet below land surface, of the point where the interval of hole begins, or open interval begins. Attribute: Depth to Top of Casing Definition: Enter the depth, in feet below land surface, to the point where the section of the casing begins. | Attribute: Well Screen Depth to Top Definition: Feet below ground surface Well Screen Depth to Top Date Measured | Attribute: Top of Perforated Interval Definition: Top of perforated interval (depth). |
| Depth to Bottom of Screen (1)…(10) | Attribute: Depth to Bottom of Screen (1)…(10) Definition: Depth to the bottom of a screened interval below ground surface. Can include up to 10 different screened intervals. Units: feet below RPE or GSE Reporting Accuracy: Tenth of a foot or greater. | Attribute: Bottom Perforation Definition: Depth below ground surface to the bottom of a perforation interval measured in feet. | Attribute: Screen Interval Bottom Definition: Bottom screen interval. | Not reported. | Attribute: Depth to Bottom of Interval Definition: Enter the depth, in feet below land surface, of the point where the interval of hole ends or open interval ends. Attribute: Depth to Bottom of Casing Definition: Enter the depth, in feet below land surface, to the point where the section of the casing ends. | Attribute: Well Screen Depth to Bottom Definition: Feet below ground surface. Well Screen Depth to Bottom Date Measured | Attribute: Bottom of Perforated Interval Definition: Bottom of perforated interval (depth). |
| Well Casing Material | Attribute: Well Casing Material Definition: Material that the well casing is made from. List of Options: Acrylonitrile butadiene styrene plastic Brick Concrete Copper PTFE plastic Fiberglass Galvanized iron Fiberglass plastic Wrought iron Fiberglas epoxy PVC plastic (threaded) Glass Other Metal PVC plastic (glued) PVC plastic FEP plastic Rock or stone Uncoated steel Tile Coated steel Stainless steel Wood Carbon steel Other Stainless-steel 304 | Not reported. | Not reported. | Not reported. | Attribute: Casing Material Definition: Enter the code that best indicates the material from which the casing is made List of Options: A (ABS) B (brick) C (concrete) D (copper) E (PTFE) F (fiberglass) G (galv. iron) H (fiberglass plastic) I (wrought iron) J (fiberglass epoxy) K (PVC threaded) L (glass) M (other metal) N (PVC glued) P (PVC or plastic) Q (FEP) R (rock or stone) S (steel) T (tile) U (coated steel) V (stainless steel) W (wood) X (steel carbon) Y (steel galvanized) Z (other material) 4 (stainless 304) 6 (stainless 316) | Well Casing Material | Casing Material Type |
| Well Casing Diameter | Attribute: Well Casing Diameter Definition: Inside diameter of the well casing. Units: Inches | Not reported. | Not reported. | Not reported. | Attribute: Diameter of Casing Definition: Nominal diameter in inches of the section of casing. | Attribute: Well Casing Size Definition: Inches | Attribute: Casing Diameter Definition: Diameter of the well casing. |
| Well Slot Size | Attribute: Well Slot Size Definition: Size of well screen individual perforations, or slots, or the mesh size of the screen. Units: Inches Reporting Accuracy: Thousandth of an inch | Not reported. | Not reported. | Not reported. | Attribute: Width of Opening Definition: The short dimension, in inches, of the individual perforations or slots, or the mesh size of screens | Attribute: Well Slot Size Definition: Inches | Slot Size |
| Well Status | Attribute: Well Status Definition: Status of the well. List of Options: Planned Active Inactive Abandoned No access Destroyed Other | Attribute: Well Status Definition: Status of well. Assumed to be active. | Attribute: Well Status | Attribute: Status Definition: Well current status List of Options: ACT INACT DRY NOACC AB DEST | Not reported. | Not reported. | Not reported. |
| Well Use Type | Attribute: Well Use Type Definition: Primary use of the well. List of Options: Municipal Agricultural Domestic Monitoring Industrial Stockwatering Public supply Unknown Other Irrigation | Attribute: Well Use Type Definition: select the description of the well’s use from the dropdown menu List of Options: Monitoring Industrial Irrigation Domestic Stockwatering Unknown Other Public supply | Well Use | Not reported. | Attribute: Use of Water Definition: Enter the code that best describes the primary use of water from the site. | Not reported. | Attribute: Planned Use/Former Use Definition: Planned well use, or former well use. |
| Well Completion Structure | Attribute: Well Completion Structure Definition: Description of well completion structure. List of Options: Single well Nested/multi-completion well | Attribute: Well Completion Type Definition: Identifies the installation of a well List of Options: Single Well Part of a nested/‌multi-completion well | Attribute: Well Completion Type Definition: Identifies the installation of a well List of Options: Single Well Part of a nested/‌multi-completion well | Not reported. | Not reported. | Not reported. | Casing Number |
| Well Completion Report Number | Attribute: Well Completion Report Number Definition: The Department of Water Resources Well Completion Report Number (OSWCR Legacy Log Number). | Attribute: Well Completion Report Number Definition: The Department of Water Resources Well Completion Report Number (OSWCR Legacy Log Number). | Well Completion Report # | Not reported. | Not reported. | Well Completion Report Number | Attribute: WCR Number Definition: Well Completion Report Number assigned by OSWCR. |
| Well Completion Date | Attribute: Well Completion Date Definition: Date that the well was completed. Date Format: MM/DD/YYYY | Not reported. | Not reported. | Not reported. | Attribute: Date of First Construction Definition: enter date on which construction began at the site. MM/DD/YYYY. Attribute: Date of Completed Construction Definition: Enter the date on which all work at the site was completed. MM/DD/YYYY. | Attribute: Well Construction Date Definition: MM/DD/YYYY | Attribute: Date Work Ended Definition: Date on which work on well ended, converted from text to date. |
| Well Location Description | Attribute: Well Location Description Definition: Description of the location of the well and any other notes used to identify the well’s location. | Attribute: Well Location Description Definition: Description of the location of the well and any other notes used to identify the well’s location. | Well Location Desc | Not reported. | Not reported. | Not reported. | Attribute: Well Location Definition: Address where the well is located. |
| Groundwater Basin | Attribute: Groundwater Basin Name Definition: Name of groundwater basin the well is located within. | Attribute: Principal Aquifer Definition: Local identification of the principal aquifer that the well monitors | Associated Basin | Not reported. | Attribute: Primary Aquifer Definition: eight-character code that best identifies the primary aquifer unit from which the water is obtained. | Not reported. | Not reported. |
| County | Attribute: County Name Definition: Name of the County the well is located within. | Attribute: County Definition: County the well is located within. | County | Not reported. | Attribute: County Definition: County Name, State of where the site is located. | Not reported. | Attribute: County Name Definition: County where the well is located. |
| SGMA Monitoring Network | Attribute: SGMA Monitoring Network Definition: The monitoring network that the well is associated with under a Groundwater Sustainability Plan. List of Options: Monitoring well Representative monitoring well Other | Attribute: Monitoring Network Type Definition: Specify whether the well is a part of the groundwater sustainability plan’s monitoring network or representative monitoring network. | Data submittals for this can be chosen from a list: CASGEM Voluntary | Not reported. | Not reported. | Not reported. | Not reported. |
| Additional Comments | Attribute: Additional Comments Definition: Any additional comments concerning the well characteristics. | Attribute: Additional Comments Definition: Provide either any additional comments concerning the well or required comments if ‘other’ was selected from any of the dropdown menus. | Additional Comments | Not reported. | Not reported. | Not reported. | Attribute: Other Observations Definition: Other observations noted on Well Completion Report. |

CASGEM = California Statewide Groundwater Elevation Monitoring

FEP = fluorinated ethylene propylene

GeoTracker = GeoTracker Data Management System

GNSS = Grade Global Navigation Satellite System

GPS = global positioning system

GSE = ground surface elevation

HPGN = High-precision Geodetic Network

LSD = land surface datum

NAD83 = North American Datum of 1983

NAVD88 = North American Vertical Datum of 1988

NGVD29 = North Geodetic Vertical Datum of 1929

PVC = polyvinyl chloride
PST = Pacific Standard Time

PTFE = polytetrefluoroethylene

RPE = reference point elevation

SAFER= Safe and Affordable Funding for Equity and Resilience Program

SGMA = Sustainable Groundwater Management Act

USGS = US Geologic Survey

WAAS = Wide Area Augmentation System